Unregulated Bus

Battery-dominated bus model in Satellite Energy Balance

Inputs, calculations, and outputs

# 1. What the model represents

The unregulated bus in this app is the Battery-dominated bus architecture. The battery connects directly to the spacecraft bus, so the bus voltage is the battery terminal voltage. That voltage moves with state of charge and with the current flowing into or out of the pack.

Bus architecture is separate from solar regulation. The Power system panel has two controls: Bus architecture, which may be Regulated or Battery-dominated, and Solar regulation, which may be MPPT or Direct Energy Transfer. Battery-dominated is the default architecture. Choosing Direct Energy Transfer describes the solar power path; it does not by itself make the bus unregulated.

The solar-array calculation still produces generated power from illumination, panel area, and the configured efficiencies. Path efficiency then scales that power before it reaches the bus. The present model does not solve a solar-array current-voltage curve at the battery voltage.

# 2. Default 8-series lithium-ion pack

The default voltage parameters represent a typical unregulated 28 V lithium-ion bus made from eight cells in series. Empty and full open-circuit voltages use 3.0 V and 4.2 V per cell. At 50 percent state of charge the open-circuit voltage is 28.8 V, the midpoint of that range.

| Parameter | Per cell | 8-cell pack |
| --- | --- | --- |
| Empty open-circuit voltage | 3.0 V | 24.0 V |
| Full open-circuit voltage | 4.2 V | 33.6 V |
| Open-circuit voltage at 50% SOC | 3.6 V | 28.8 V |
| Equivalent series resistance | — | 0.08 Ω |
| Minimum usable bus voltage | 3.0 V | 24.0 V |

The rest of the default power system remains a 1,000 Wh battery at 100 percent initial state of charge, a 30 percent depth-of-discharge reporting limit, 95 percent charge efficiency, 98 percent discharge efficiency, a 500 W maximum charge rate, 90 percent path efficiency, and MPPT solar regulation.

# 3. How each time step is calculated

At every simulation step the bus model receives generated array power, load power, and the mission time step. It updates stored energy and, in battery-dominated mode, terminal voltage and current.

## Power balance

P_delivered = P_generated × path efficiency

P_net = P_delivered − P_load

Positive net power is a charging surplus. Negative net power is a load deficit that the battery must supply.

## Open-circuit voltage

Open-circuit voltage changes linearly with the state of charge at the beginning of the step. The state-of-charge fraction is stored energy divided by capacity, limited to the range 0 to 1.

V_oc = V_empty + SOC_fraction × (V_full − V_empty)

## Charging

Accepted charge power is the smallest of the available surplus, the configured maximum charge rate, and the power the pack can accept at the full-pack voltage ceiling. With pack resistance R, the terminal voltage and terminal power are:

V_bus = V_oc + I × R

P_terminal = I × (V_oc + I × R)

The current that produces a requested terminal power P is:

I = 2P / (V_oc + √(V_oc² + 4RP))

The energy stored in the battery uses the open-circuit voltage, so the ohmic loss is excluded from stored energy:

P_stored = V_oc × I × charge efficiency

Surplus power above the accepted charge power is reported as shunt power. As the pack approaches 100 percent state of charge, open-circuit voltage approaches the full-pack voltage and the allowed charge current tapers toward zero.

## Discharging

The requested discharge power is the magnitude of a negative net power.

P_needed = −P_net

Current is limited so the terminal voltage remains at or above the configured minimum bus voltage:

I_voltage = (V_oc − V_min) / R

P_voltage = V_min × I_voltage

Demand above that voltage-limited power is reported as voltage-limited power. This electrical limit is calculated from voltage and resistance. Stored energy imposes a second limit: the battery cannot supply more chemical energy than it currently holds. The delivered current is the smaller of the two limits.

For the current that is actually delivered:

V_bus = V_oc − I × R

P_terminal = I × (V_oc − I × R)

P_chemical = (V_oc × I) / discharge efficiency

Unmet load is requested load power minus power actually delivered at the bus. It includes demand blocked by the minimum voltage and demand that remains after the battery is empty. Voltage-limited energy can therefore be zero while unmet energy is still positive, when the pack voltage could have supported the load but the stored energy could not.

## Signs and bounds

Battery current is positive while charging and negative while discharging. Battery terminal power is positive into the battery and negative out of the battery. With zero net power, current is zero and bus voltage equals the open-circuit voltage. Stored energy is kept between zero and the configured capacity.

# 4. Input parameters

The tables use the labels shown in the sidebar. The JSON names are the fields written by Download config.

## Power system

| Sidebar input | JSON field | Default | Effect on the unregulated bus |
| --- | --- | --- | --- |
| Bus architecture | power_system.bus_architecture | Battery-dominated | Selects the unregulated bus. The saved value is battery_dominated. |
| Solar regulation | power_system.regulation | MPPT | Recorded as mppt or direct_energy_transfer. It does not change the bus-voltage equations. |
| Path efficiency | power_system.path_efficiency | 0.90 | Scales generated power before the bus power balance. Sidebar range: 0.30 to 1.00. |

## Battery energy

| Sidebar input | JSON field | Default | How it is used |
| --- | --- | --- | --- |
| Capacity (Wh) | battery.capacity_wh | 1,000 Wh | Energy at 100% state of charge. Sidebar range: 1 to 100,000 Wh. |
| Initial SOC (%) | battery.initial_soc_percent | 100% | Starting stored energy and starting open-circuit voltage. |
| Max depth-of-discharge (%) | battery.max_dod_percent | 30% | Reporting limit for the verdict and SOC chart. Discharge can continue below it. |
| Charge efficiency | battery.charge_efficiency | 0.95 | Fraction of chemical charge power retained as stored energy. |
| Discharge efficiency | battery.discharge_efficiency | 0.98 | Relates delivered electrical power to chemical energy removed. |
| Max charge rate (W) | battery.max_charge_rate_w | 500 W | Upper bound on accepted charge power, together with the voltage ceiling. |

## Pack voltage

These four inputs appear under Battery only while Bus architecture is Battery-dominated. They describe one equivalent pack, using the eight-cell defaults above.

| Sidebar input | JSON field | Default | Meaning |
| --- | --- | --- | --- |
| Empty-pack OCV (V) | battery.empty_ocv_v | 24.0 V | Open-circuit voltage at 0% state of charge. |
| Full-pack OCV (V) | battery.full_ocv_v | 33.6 V | Open-circuit voltage at 100% state of charge and the charge-voltage ceiling. |
| Pack internal resistance (Ω) | battery.internal_resistance_ohm | 0.080 Ω | Series resistance used for charge rise and discharge sag. |
| Minimum bus voltage (V) | battery.minimum_bus_voltage_v | 24.0 V | Lowest terminal voltage at which the model continues to serve load. |

The app requires full-pack open-circuit voltage to be greater than empty-pack open-circuit voltage, and it requires minimum bus voltage to be no higher than the full-pack open-circuit voltage. An invalid sidebar entry produces an error and the previous valid relationship remains in the configuration.

## Values supplied by the rest of the simulation

| Value | Source | Role |
| --- | --- | --- |
| Generated power | Solar-array model, column gen_w | Array output before path efficiency. |
| Load power | Load schedule, column load_w | Power demanded from the bus at each step. |
| Time step | Mission panel, Time step (s) | Integration interval. The sidebar default is 30 seconds. |

# 5. Outputs

Battery-dominated mode adds voltage and current results to the energy results used by every architecture. In regulated mode the voltage and current columns are empty.

## Time series

| Column | Unit | Meaning |
| --- | --- | --- |
| bus_voltage_v | V | Battery terminal voltage, which is the unregulated bus voltage. |
| battery_current_a | A | Pack current. Positive charges the battery; negative discharges it. |
| battery_power_w | W | Terminal power. Positive enters the battery; negative leaves it. |
| voltage_limited_w | W | Load power that would pull the bus below the minimum voltage. |
| soc_wh | Wh | Stored battery energy after the step. |
| soc_percent | % | Stored energy as a percentage of capacity. |
| shunt_w | W | Surplus power that the battery does not accept. |
| unmet_w | W | Requested load power that is not delivered. |

These columns are included in the simulation results. Download results (CSV) writes every results column except the UTC timestamp.

## Summary tab

| Displayed result | Meaning |
| --- | --- |
| Bus architecture | Shows Battery-dominated when the unregulated model is active. |
| Energy in, Energy out | Integrated generated energy and load energy over the run. |
| Shunted | Integrated surplus energy that was not stored. |
| Unmet | Integrated load energy that was not delivered. |
| Min SOC | Lowest state of charge during the run. |
| Max observed DOD | 100% minus the minimum state of charge. |
| Min bus voltage | Lowest calculated terminal voltage. Blank for a regulated bus. |
| Voltage-limited | Integrated energy associated with voltage-limited power. |

The visible verdict checks whether average generation covers average load, whether the observed depth of discharge stays within its limit, whether unmet energy is effectively zero, and whether the battery recovers above 95 percent state of charge on the last full orbit. The overall pass result uses the first three of those checks. A separate internal flag records whether bus voltage stayed at or above its minimum; the Summary tab presents that condition through minimum bus voltage, voltage-limited energy, and unmet energy.

## Battery tab

With Battery-dominated selected, the Battery tab plots four aligned time series:

State of charge, with a horizontal line at the depth-of-discharge limit.

Bus voltage, with a horizontal line at the minimum bus voltage.

Battery current, using the charge-positive and discharge-negative convention.

Shunt power, unmet load, and voltage-limited power.

## Saved configuration

Download config (JSON) preserves the architecture and pack parameters. A battery-dominated 8-series configuration contains:

power_system.bus_architecture = "battery_dominated"

battery.empty_ocv_v = 24.0

battery.full_ocv_v = 33.6

battery.internal_resistance_ohm = 0.08

battery.minimum_bus_voltage_v = 24.0

Older configuration files that omit these fields load with the same defaults, including the battery-dominated architecture.

# 6. Modeling boundaries

The implementation is a pack-level energy and terminal-voltage model. Open-circuit voltage follows one straight line from empty to full. Resistance is constant. Charge efficiency, discharge efficiency, and internal resistance account for the difference between terminal energy and stored chemical energy.

The model includes charge-rate limiting, charge taper at the full-pack voltage, discharge limiting at the minimum bus voltage, and energy limiting at zero state of charge. The depth-of-discharge setting remains a reporting threshold.

The model does not include a measured cell curve, temperature dependence, a separate charge or bus regulator, or a solar-array current-voltage solution clamped to the instantaneous battery voltage. Those effects would be additional models around this bus calculation.